For my student project, I examined factors that potentially affect the fuel efficiency of cars.  The data was found on http://www.umass.edu/statdata/statdata/stat-rmult.html and originally came from Consumer Reports:  The 1993 Cars - Annual Auto Issue (April 1993) and PACE New Car & Truck 1993 Buying Guide (1993).

The raw data contained 26 variables so I chose the ones that I predicted would have the most effect on fuel efficiency.

The variables are as follows:

Y = Highway Miles perGallon

X1 = Engine Size (liters)

X2 = Maximum Horsepower

X3 = Fuel Tank Capacity

X4 = Passenger Capacity

X5 = Length (inches)

X6 = Weight

D1 = Domestic (1 = US manufacturer, 0 = non-US manufacturer)

Model 1

The full regression equation is:

Y =  + 1X1 + 2X2 + 3X3 + 4X4 + 5X5 + 6X6 + 1D1

I ran regression analysis in Excel to determine the values of the coefficients:

Y = 41.4761 + 1.2513X1 – 0.0125X2 - 0.4754X3 – 0.3181X4 + 0.1069X5 – 0.0075X6 – 1.5565X7

R2 = 0.7265 and the adjusted R2 = 0.7040 which means only 70.4% of the variation of fuel efficiency is captured by this regression model.

Passenger capacity has a high p-value and a t-statistic with an absolute value less than 2, which indicates that it doesn’t have much significance in this model.  I will remove the X4variable.  Horsepower also has a high p-value, and it’s correlation to engine size is high (0.7118) so multicollinearity appears to be an issue in this model.  I will remove the X2 variable as well.

Model 2

The new model is:

Y =  + 1X1 + 3X3 + 5X5 + 6X6 + 1D1

Regression Analysis gives me the following coefficients:

Y = 40.6410 + 1.0713X1 - 0.5069X3 + 0.1118X5 – 0.0084X6 – 1.4862D1

R2 is now 0.7235 and adjusted R2is 0.7076, a small improvement but not much.

I looked at the correlation between each pair of variables.  There is high correlation between all pairs of variables except whether or not the vehicle is domestic.  Fuel tank capacity and weight have the highest correlation (0.894) so I will remove fuel tank capacity.  Then I will run the analysis with each of the remaining variables paired with the domestic dummy variable.

Model 3

Engine size plus domestic variable:

Y =  + 1X1 + 1D1

Regression Analysis gives me the following coefficients:

Y = 37.6801 – 3.3856X1+ 0.8482D1

The adjusted R2, at 0.3849, is much worse in this model.

Model 4

Length plus domestic variable:

Y =  + X5 + 1D1

Regression Analysis gives me the following coefficients:

Y = 65.7077 – 0.2004X5+ 0.1724D1

The adjusted R2 dropped even lower with this model.

Model 5

Weight plus domestic variable:

Y =  + X6+ 1D1

Regression Analysis gives me the following coefficients:

Y = 51.5969 – 0.0073X5-0.0938D1

The adjusted R2 in this model is 0.6496 so weight appears to account for most of the variation in Y captured by the first model.

In Models 3-5, the domestic variable has a high p-value and thus there is a high probability that the null hypothesis for this variable (1 = 0) is true.  So I will run one more model with only weight.

Model 6

Weightonly:

Y =  + X6

Regression Analysis gives me the following coefficients:

Y = 51.6014 – 0.0073X5

The adjusted R2 in this model is slightly higher than in Model 5, and the p-value for weight is sufficiently small.

The original model (Model 1) is far from ideal because multicollinearity is present and the model does not account for all of the variation in fuel efficiency.  However, of the variables I started with, weight appears to be the best indicator of fuel efficiency.