Driveline Mechanics and Fuel Economy

Introduction:

The aim of this paper is to determine the impact of vehicular drivelines on fuel economy (miles per gallon or MPG) using explanatory variables for the various components necessary to get energy from an engine to the driving wheels.  The data utilized is for 2012 models years and available from the US EPA (http://www.fueleconomy.gov/feg/download.shtml).

Objectives of the Study:

- Determine the key driveline components that impact fuel economy.

- Find a regression model that best estimates fuel economy.

Data:

The initial data set is composed of 900 entries with 126 different explanatory variables potentially available.  Upon review, electric cars were removed from the data set and the description fields were pared down to 8 fields.  Electric cards were removed from the data set, providing a sample of 896 vehicles for the regression.  For the 896 observations, the data provides the following variables:

a. Engine Displacement (Liters)
b. Number of Cylinders 
c. Number of Gears
d. Air Aspiration Method, coded as indicated below.
e. Transmission Type, coded as indicated below.
f. Transmission Lockup Present, coded as indicated below.
g. Driveline Type, coded as indicated below.
h. Fuel Economy (Y)

| Air Aspiration Method | Value |  | Transmission Lock Up Present | Value |
| --- | --- | --- | --- | --- |
| Supercharged | 0 |  | No | 0 |
| Naturally Aspirated | 1 |  | Yes | 1 |
| Turbocharged | 2 |  |  |  |
| Transmission Type | Value |  | Driveline Type | Value |
| Selectable Continuously Variable | 0 |  | 2-Wheel Drive, Rear | 0 |
| Automatic | 1 |  | All Wheel Drive | 1 |
| Semi-Automatic | 2 |  | 2-Wheel Drive, Front | 2 |
| Manual | 3 |  | 4-Wheel Drive | 3 |
| Continuously Variable | 4 |  | Part-time 4-Wheel Drive | 4 |
| Automated Manual | 5 |  |  |  |

A sample of the first 10 rows of data is:

| Entry | Engine Displacement | Number of Cylinders | Air Aspiration Method | Transmission Type | Number of Gears | Transmission Lockup Present | Driveline Type | Fuel Economy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5.9 | 12 | 1 | 3 | 6 | 0 | 0 | 13 |
| 2 | 4.7 | 8 | 1 | 5 | 6 | 0 | 0 | 16 |
| 3 | 4.7 | 8 | 1 | 3 | 6 | 0 | 0 | 15 |
| 4 | 4.7 | 8 | 1 | 5 | 7 | 0 | 0 | 16 |
| 5 | 4.2 | 8 | 1 | 5 | 6 | 0 | 1 | 16 |
| 6 | 4.2 | 8 | 1 | 3 | 6 | 0 | 1 | 14 |
| 7 | 5.2 | 10 | 1 | 5 | 6 | 1 | 1 | 15 |
| 8 | 5.2 | 10 | 1 | 3 | 6 | 0 | 1 | 14 |
| 9 | 4.2 | 8 | 1 | 5 | 6 | 0 | 1 | 16 |
| 10 | 4.2 | 8 | 1 | 3 | 6 | 0 | 1 | 14 |

Preliminary Analysis:

A linear regression assumes the error for each variable follows a standard normal with constant variance and are independent of one another.  One way to test for skew within the data is to analyze the ratio of the hinge spreads.  This analysis produces the following results:

|  | Engine Displacement | Number of Cylinders | Air Aspiration Method | Transmission Type | Number of Gears | Transmission Lockup Present | Driveline Type | Fuel Economy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Hinge | 4.60 | 8.00 | 1.00 | 3.00 | 6.00 | 1.00 | 2.00 | 24.00 |
| Median | 3.50 | 6.00 | 1.00 | 2.00 | 6.00 | 1.00 | 1.00 | 21.00 |
| Lower Hinge | 2.40 | 4.00 | 1.00 | 1.00 | 5.00 | 1.00 | 0.00 | 17.00 |
| Upper Hinge Distance | 1.10 | 2.00 | 0.00 | 1.00 | 0.00 | 0.00 | 1.00 | 3.00 |
| Lower Hinge Distance | 1.10 | 2.00 | 0.00 | 1.00 | 1.00 | 0.00 | 1.00 | 4.00 |
| Ratio | 1.00 | 1.00 | 0.00 | 1.00 | 0.00 | 0.00 | 1.00 | 0.75 |

Review of the hinge distances indicates skew exists within the Fuel economy Data (Y) with ratio of 0.75.  However, analysis of a histogram and box plot indicate outliers are responsible for producing a heavy tail.  Therefore, no transformation is needed on the dependent variable Fuel economy.

Regression Analysis:

The method to determine a suitable regression model will involve a backwards process, beginning with a regression with all variables and removing them until all explanatory variables are deemed significant.

Regression Model 1: 7 explanatory variables

| SUMMARY OUTPUT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |  |  |  |
| Multiple R | 0.8372621 |  |  |  |  |  |  |  |
| R Square | 0.7010078 |  |  |  |  |  |  |  |
| Adjusted R Square | 0.6986508 |  |  |  |  |  |  |  |
| Standard Error | 2.9424354 |  |  |  |  |  |  |  |
| Observations | 896 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |  |  |  |
| Regression | 7 | 18025.601 | 2575.0858 | 297.42525 | 8.12E-228 |  |  |  |
| Residual | 888 | 7688.2384 | 8.6579261 |  |  |  |  |  |
| Total | 895 | 25713.839 |  |  |  |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% | Upper 95% | Lower 95.0% | Upper 95.0% |
| Intercept | 33.559862 | 0.8294184 | 40.461921 | 1E-203 | 31.932013 | 35.187711 | 31.932013 | 35.187711 |
| Engine Displacement | -2.8392948 | 0.1874538 | -15.14664 | 2.862E-46 | -3.2071989 | -2.4713907 | -3.2071989 | -2.4713907 |
| Number of Cylinders | -0.1257501 | 0.1353306 | -0.9292068 | 0.3530344 | -0.3913551 | 0.1398549 | -0.3913551 | 0.1398549 |
| Air Aspiration Method | -0.4949485 | 0.2424021 | -2.0418493 | 0.0414615 | -0.9706962 | -0.0192007 | -0.9706962 | -0.0192007 |
| Transmission Type | 0.4535629 | 0.1303397 | 3.4798532 | 0.0005261 | 0.1977532 | 0.7093726 | 0.1977532 | 0.7093726 |
| Number of Gears | -0.3077397 | 0.0802601 | -3.8342829 | 0.0001348 | -0.4652612 | -0.1502182 | -0.4652612 | -0.1502182 |
| Transmission Lockup Present | -0.2961846 | 0.288499 | -1.0266397 | 0.3048697 | -0.862404 | 0.2700349 | -0.862404 | 0.2700349 |
| Driveline Type | 0.089101 | 0.0976922 | 0.9120589 | 0.3619852 | -0.1026335 | 0.2808356 | -0.1026335 | 0.2808356 |

The initial regression model has an adjusted R^2 of 0.699, which means 69.9% of the variability in Fuel economy is explained by this model.  The F- value is 297.42 and the standard error is 2.94.  Looking at the P-values, Engine Displacement, Transmission Type, and Number of Gears are < 0.001.  The significant importance of these explanatory variables means they should remain.  Driveline Type has the largest P-value of 0.36, which is significantly high. For the next step in the iteration process, this variable will be removed.

Regression Model 2: 6 explanatory variables

| SUMMARY OUTPUT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |  |  |  |
| Multiple R | 0.8370948 |  |  |  |  |  |  |  |
| R Square | 0.7007277 |  |  |  |  |  |  |  |
| Adjusted R Square | 0.6987078 |  |  |  |  |  |  |  |
| Standard Error | 2.9421571 |  |  |  |  |  |  |  |
| Observations | 896 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |  |  |  |
| Regression | 6 | 18018.399 | 3003.0665 | 346.9231 | 6.33E-229 |  |  |  |
| Residual | 889 | 7695.4405 | 8.6562885 |  |  |  |  |  |
| Total | 895 | 25713.839 |  |  |  |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% | Upper 95% | Lower 95.0% | Upper 95.0% |
| Intercept | 33.839061 | 0.7707875 | 43.901933 | 8.1E-225 | 32.326286 | 35.351837 | 32.326286 | 35.351837 |
| Engine Displacement | -2.8496767 | 0.1870901 | -15.23157 | 1.011E-46 | -3.2168665 | -2.4824868 | -3.2168665 | -2.4824868 |
| Number of Cylinders | -0.1371258 | 0.1347418 | -1.0176933 | 0.3091006 | -0.401575 | 0.1273233 | -0.401575 | 0.1273233 |
| Air Aspiration Method | -0.5025893 | 0.2422343 | -2.0748061 | 0.0382917 | -0.9780071 | -0.0271715 | -0.9780071 | -0.0271715 |
| Transmission Type | 0.448877 | 0.130226 | 3.4469066 | 0.0005936 | 0.1932907 | 0.7044633 | 0.1932907 | 0.7044633 |
| Number of Gears | -0.3143156 | 0.079928 | -3.932485 | 9.061E-05 | -0.4711852 | -0.1574461 | -0.4711852 | -0.1574461 |
| Transmission Lockup Present | -0.2970248 | 0.2884703 | -1.0296549 | 0.3034521 | -0.8631869 | 0.2691373 | -0.8631869 | 0.2691373 |

Removing the Driveline Type provides an unchanged adjusted 0.699R^2 while the F-value has increased and the standard error has dropped, albeit slightly.  Analysis of the P-values indicates the variable Number of Cylinders is least significant and should be removed for the next iteration.

Regression Model 3: 5 explanatory variables

| SUMMARY OUTPUT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |  |  |  |
| Multiple R | 0.8368865 |  |  |  |  |  |  |  |
| R Square | 0.700379 |  |  |  |  |  |  |  |
| Adjusted R Square | 0.6986958 |  |  |  |  |  |  |  |
| Standard Error | 2.9422161 |  |  |  |  |  |  |  |
| Observations | 896 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |  |  |  |
| Regression | 5 | 18009.433 | 3601.8867 | 416.08389 | 4.94E-230 |  |  |  |
| Residual | 890 | 7704.4058 | 8.6566357 |  |  |  |  |  |
| Total | 895 | 25713.839 |  |  |  |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% | Upper 95% | Lower 95.0% | Upper 95.0% |
| Intercept | 33.850288 | 0.770724 | 43.920116 | 4.98E-225 | 32.33764 | 35.362937 | 32.33764 | 35.362937 |
| Engine Displacement | -3.0240826 | 0.0750597 | -40.289018 | 7.92E-203 | -3.1713973 | -2.8767679 | -3.1713973 | -2.8767679 |
| Air Aspiration Method | -0.545791 | 0.2384905 | -2.2885233 | 0.0223406 | -1.0138602 | -0.0777217 | -1.0138602 | -0.0777217 |
| Transmission Type | 0.4270841 | 0.1284559 | 3.3247522 | 0.0009211 | 0.1749723 | 0.6791959 | 0.1749723 | 0.6791959 |
| Number of Gears | -0.3348892 | 0.0773307 | -4.3306086 | 1.656E-05 | -0.486661 | -0.1831173 | -0.486661 | -0.1831173 |
| Transmission Lockup Present | -0.302585 | 0.2884243 | -1.0490968 | 0.2944184 | -0.8686561 | 0.263486 | -0.8686561 | 0.263486 |

Reducing the model by one variable still leaves the adjusted R^2 essentially unchanged at 0.699.  The standard error increased slightly, but the F-value increased substantial indicating the model gaining significance.  P-values indicate that Transmission Lockup should be removed for the next iteration.

Regression Model 4: 4 explanatory variables

| SUMMARY OUTPUT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |  |  |  |
| Multiple R | 0.8366651 |  |  |  |  |  |  |  |
| R Square | 0.7000085 |  |  |  |  |  |  |  |
| Adjusted R Square | 0.6986617 |  |  |  |  |  |  |  |
| Standard Error | 2.9423822 |  |  |  |  |  |  |  |
| Observations | 896 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |  |  |  |
| Regression | 4 | 17999.906 | 4499.9765 | 519.77103 | 3.53E-231 |  |  |  |
| Residual | 891 | 7713.9333 | 8.6576132 |  |  |  |  |  |
| Total | 895 | 25713.839 |  |  |  |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% | Upper 95% | Lower 95.0% | Upper 95.0% |
| Intercept | 33.42758 | 0.6570504 | 50.875218 | 8.46E-266 | 32.138034 | 34.717127 | 32.138034 | 34.717127 |
| Engine Displacement | -3.0381899 | 0.0738495 | -41.140272 | 3.54E-208 | -3.1831293 | -2.8932506 | -3.1831293 | -2.8932506 |
| Air Aspiration Method | -0.546147 | 0.2385037 | -2.2898893 | 0.0222607 | -1.0142415 | -0.0780526 | -1.0142415 | -0.0780526 |
| Transmission Type | 0.5029885 | 0.1061479 | 4.7385633 | 2.504E-06 | 0.2946594 | 0.7113175 | 0.2946594 | 0.7113175 |
| Number of Gears | -0.3176724 | 0.0755736 | -4.2034849 | 2.893E-05 | -0.4659954 | -0.1693494 | -0.4659954 | -0.1693494 |

Once again, the adjusted R^2 did not move significantly as a high P-value explanatory variable was removed.  The standard increased slightly and the F-value increased significantly, as seen previously.  P-values once again indicate that Air Aspiration Method is insignificant and should be removed for the next iteration.

Regression Model 5: 3 explanatory variables

| SUMMARY OUTPUT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regression Statistics |  |  |  |  |  |  |  |  |
| Multiple R | 0.8356094 |  |  |  |  |  |  |  |
| R Square | 0.698243 |  |  |  |  |  |  |  |
| Adjusted R Square | 0.6972281 |  |  |  |  |  |  |  |
| Standard Error | 2.949373 |  |  |  |  |  |  |  |
| Observations | 896 |  |  |  |  |  |  |  |
| ANOVA |  |  |  |  |  |  |  |  |
|  | df | SS | MS | F | Significance F |  |  |  |
| Regression | 3 | 17954.509 | 5984.8363 | 688.00705 | 1.69E-231 |  |  |  |
| Residual | 892 | 7759.3303 | 8.6988008 |  |  |  |  |  |
| Total | 895 | 25713.839 |  |  |  |  |  |  |
|  | Coefficients | Standard Error | t Stat | P-value | Lower 95% | Upper 95% | Lower 95.0% | Upper 95.0% |
| Intercept | 32.863121 | 0.6105057 | 53.82934 | 1.97E-282 | 31.664926 | 34.061316 | 31.664926 | 34.061316 |
| Engine Displacement | -3.0008694 | 0.0721998 | -41.563398 | 7.13E-211 | -3.1425707 | -2.8591681 | -3.1425707 | -2.8591681 |
| Transmission Type | 0.4852994 | 0.1061179 | 4.5732073 | 5.481E-06 | 0.2770294 | 0.6935693 | 0.2770294 | 0.6935693 |
| Number of Gears | -0.3473153 | 0.0746335 | -4.6536086 | 3.756E-06 | -0.4937931 | -0.2008375 | -0.4937931 | -0.2008375 |

For this particular model, the adjusted R^2 has not changed significantly from the previous model along with the standard error, while the F-value increased significantly.  The P-values for all explanatory variables are now < 0.001, indicating all of the explanatory variables are significant.

The best fit equation for modeling fuel economy is

Fuel economy = -3.00*Engine_Displacement + 0.485*Trans_Type - 							0.347*#_of_Gears + 32.86

The final adjusted R^2 is 0.6972281 which reflects 69.7% of the variation in fuel economy is explained by the variables selected.

Summary of Findings and Future Work:

The analysis performed in this paper shows that driveline components do have an impact on fuel economy.  In particular, the engine displacement, transmission type, and the number of gears are significantly related to fuel economy.  Furthermore, the engine displacement has the largest impact on fuel economy, with a coefficient of -3, indicating as engine size increases, fuel economy drops.  An increased number of gears provides a better fuel economy while the continuous variable and automated manual transmissions provide better fuel economy compared to a Selectable Continuously Variable or Automatic Transmission.  It also has shown that the number of cylinders, a transmission lockup, air aspiration method, and driveline type have no significant impact on the predicted fuel economy.

The model takes a simplistic linear viewpoint.  One way the model could be improved is the inclusion of dummy variables and the interactions between all variables to increase the potential significance and corresponding R^2.  The complicating factor with this method is the large number of initial variables that will be generated, which will require substantial computing power.

Future work could include the implementation of aerodynamic work to correspond with the mechanical driveline data analyzed in this paper.  A intuitive statement is that a smaller, sleeker car will provide a better fuel economy than a large, boxy type vehicle.  The inclusion of total frontal area and some sort of aerodynamic coefficient could provide a higher explanation of the total variation concerning fuel economy.  Another factor to consider down the road is differences in fuel types with the emergence of hydrogen, E-85, and methane powered engines in the last few years.